SECTION 1: IDENTIFICATION OF THE MIXTURE AND OF THE COMPANY/ UNDERTAKING

1.1.	Product identifier

Commercial name	Nitrosol (30% N)

Other names	UAN solution

1.2.	Relevant identified uses of the mixture and uses advised against

Identified uses: liquid fertilizer, fertilizer for industrial uses.

1.3.	Details of the supplier of the safety data sheet

Name of manufacturer:	NITROGÉNMŰVEK Zrt.

Address:	Pétfürdő, Hősök tere 14.

8105 Pétfürdő, Pf. 450, Hungary

Telephone:	+36-88-620-100

Fax:	+36-88-620-102

E-mail:	sds@nitrogen.hu

1.4.	Emergency telephone number

National Poisons Information Service (NPIS)

NHS 111 (England), NHS 24 (Scotland) or NHS Direct (Wales) – dial 111

In Northern Ireland contact your local GP

Healthcare Professionals: UK NPIS 0344 892 0111

SECTION 2: HAZARDS IDENTIFICATION

2.1.	Classification of the mixture

Not classified according to Regulation (EC) No 1272/2008

Note: Data supporting classification are detailed in Sections 11.1 and 16.

2.2.	Label elements

Not required.

2.3.	Other hazards

- The ingredients of the product do not meet the criteria for PBT or vPvB substances and for endocrine disruptors.

- SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

3.2. 	Mixtures

Hazardous ingredient(s):

| Name | CAS number | EC number | w/w% | Registration number |
| --- | --- | --- | --- | --- |
| Ammonium nitrate | 6484-52-2 | 229-347-8 | 38-46 | 01-2119490981-27-0082 |

Classification of ammonium nitrate:

| Classification: | Ox. Sol 3, Eye Irrit. 2 |
| --- | --- |
| Hazard statements: | H272 May intensify fire; oxidiser. H319 Causes serious eye irritation. |

Other not hazardous ingredients:

| Name | CAS number | EC number | w/w% | Registration number |
| --- | --- | --- | --- | --- |
| Urea (CO(NH2)2) | 57-13-6 | 200-315-5 | 29-35 | 01-2119463277-33-0081 |

SECTION 4: FIRST AID MEASURES

4.1.	Description of first aid measures

- Skin contact

Wash the affected area with soap and water for at least 15 minutes. Remove contaminated clothing and shoes. In case of persistent irritation, obtain medical help.

- Eye contact

Flush/wash eye with plenty of water for at least 15 minutes, with occasional blinking. If necessary and if easy to do so, remove contact lenses. In case of persistent eye irritation, obtain medical help.

- Ingestion

Do not induce vomiting. Flush the mouth of the victim with water. In case of persistent sickness, obtain medical help.

- Inhalation

Remove the injured person from the exposition. Even in case of no symptoms, keep him warm and calm. If the breathing stops or in case of breathing difficulties, administer artificial respiration if qualified personnel is available. Avoid mouth to mouth resuscitation. In case of sickness, obtain medical help.

4.2.	Most important symptoms and effects, both acute and delayed

Eyes, skin: 	Redness, pain.

Ingestion:	In case of small quantities, the poisoning effect is unlikely. In case of ingestion of larger quantities may cause digestive abnormalities (abdominal pain, nausea, diarrhoea) and in extreme cases (particularly if the victim is very young), methaemoglobin formation (blue baby disease) and cyanosis (bluish discoloration of the area around the mouth) may occur.

Inhalation: 	The high airborne dust concentration may irritate the nose and the upper respiratory tract, which has symptoms like burning feeling in the throat and coughing.

4.3.     Indication of any immediate medical attention and special treatment needed

In normal cases immediate medical help is not required, but in case of persistent symptoms, obtain medical help. May cause methaemoglobin formation.

SECTION 5: FIREFIGHTING MEASURES

5.1.	Extinguishing media

Every extinguishing media can be used. Water spray is the suitable extinguishing medium.

- 5.2. 	Special hazards arising from the substance or mixture

The fertilizer is not combustible in itself, but its dried remains may promote the combustion even in the lack of air. It melts in case of heating and further heating may cause degradation which happens with the liberation of toxic nitrogen oxides and ammonia.

After the inhalation of degradation gases or degradation products, remove the injured person from the gas exposure. Even in case of no symptoms, keep him warm and calm. Give oxygen, especially if bluish discoloration can be observed around the mouth. Administer artificial respiration if the breathing has stopped. After the exposure the victim must be kept under medical surveillance for at least 48 hours, because delayed pulmonary oedema may occur.

5.3.	Advice for firefighters

Do not inhale the combustion gases (toxic). Approach the fire from down-wind.

Due to the toxic degradation and combustion products, the use of self-contained breathing apparatus is recommended, and full protective suit has to be worn.

- SECTION 6: ACCIDENTAL RELEASE MEASURES

6.1.	Personal precautions, protective equipment, and emergency procedures

Avoid contact with skin and eyes. Use the recommended PPE during the cleaning of the spillage.

6.2. 	Environmental precautions

Avoid the contamination of drains and sewage. In case of large quantities gets into sewage, surface or subsurface water, inform the respective environmental protection authority, because it may cause eutrophication.

6.3.	Methods and material for containment and cleaning up

The leak must be sealed off. The spillage should be pumped up or soaked with dry sand or soil and placed in clean, labelled containers until safe disposal.  The contaminated area or objects may be decontaminated by washing them with clean water. Do not mix with sawdust or other combustible or organic materials.

6.4. 	Reference to other sections

Recommendations regarding personal protective equipment can be found in section 8, those regarding the handling of waste van be found in section 13.

- SECTION 7: HANDLING AND STORAGE

7.1.	Precautions for safe handling

No special measures required. Avoid contamination with combustible materials (e.g. diesel fuel, grease, etc.). Its dried remains may promote the combustion.

In case of long-term handling of the product, use appropriate PPE (e.g. gloves).

7.2.	Conditions for safe storage, including any incompatibilities

- Suitable structural material of the container: can be stored in plastic cylinders, plastic drums and containers made of plastic, acid-resistant steel, or other structural material with plastic -and rubber lining (carbon steel is corroded by the product).

Keep away from heat sources and fire. In agricultural plantations ensure that the fertilizer is not stored near hay, straw, grain, diesel oil etc. Do not use open flame and do not smoke in the vicinity of the storage area.

Keep the storage environment tidy.

7.3.	Specific end use(s)

- For professional use

- packing, repacking, loading, transport

- preparing fertiliser mixtures, solutions (mixing, diluting)

Duration and frequency of use: > 4 hours/day

- drip irrigation

- outdoor foliar fertilisation (top dressing, foliar and stem fertilisation)

- greenhouse foliar fertilisation (top dressing, foliar and stem fertilisation)

Duration and frequency of use: maximum 12 hours/day; 7 days/week; 2-3 months/year

Risk reducing measures in case of professional users:

- Recommended: use automated and / or closed systems.

- Avoid the formation and inhalation of powders and inhalable particles/sprays.

- The necessary protective equipment is listed in section 8.2.2. If the mixture/solution to be produced and used contains more than 10 % ammonium nitrate and exposure cannot be excluded otherwise, eye protection is mandatory. The use of protective gloves is recommended. Wash hands thoroughly and remove work clothes after work.

- For consumer use

- dilution

- drip irrigation

- cottage, spray application (top dressing, foliar and stem fertilisation) - with hand sprayer

- greenhouse spray application (top dressing, foliar and stem fertilisation) - with hand sprayer

Duration and frequency of use: < 4 hours/day; 1-3 times/year

Risk reducing measures in case of consumers:

- Avoid the formation and inhalation of powders and inhalable particles/sprays.

- The necessary protective equipment is listed in section 8.2.2. If the mixture/solution to be produced and used contains more than 10 % ammonium nitrate and exposure cannot be excluded otherwise, eye protection is mandatory. The use of protective gloves is recommended. Wash hands thoroughly and remove work clothes after work.

(Recommended plant-specific dosage is available on the following website: ww.genezispartner.hu)

- SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

8.1.	Control parameters

8.1.1. 	Occupational exposure limit values

There are no officially established limits (EH40/2005 Workplace Exposure Limits). Maximum powder concentration recommended by ACGIH 10 mg/m3.

8.1.2. 	Recommended exposure controls

It is advisable to frequently control the concentration of dust in the work area depending on the technological stability.

8.1.3. 	Occupational exposure limits in case of generation of air-polluting material

In case of intended use of the product, no air-polluting materials are generated.

8.1.4. 	DNEL and PNEC values

For ammonium nitrate:

| DNEL (long-term) | workers | general population |
| --- | --- | --- |
| dermal | 21.3 mg/kg/day | 12.8 mg/kg/day |
| inhalation | 37.6 mg/m3 | 11.1 mg/m3 |
| ingestion | - | 12.8 mg/kg/day |

PNEC value for fresh water: 0.45 mg/l

For urea:

| DNEL (acute/long-term) | workers | general population |
| --- | --- | --- |
| dermal | 580 mg/kg/day | 580 mg/kg/day |
| inhalation | 292 mg/m3 | 125 mg/m3 |
| ingestion | - | 42 mg/kg/day |

8.1.5.  Information supporting risk management

No other data supporting risk management is available.

8.2.	Exposure controls

8.2.1.  Appropriate engineering controls

Avoid contact with eyes, skin and spilling into watercourses and drains.

8.2.2.	Personal protective equipment

In case of long-term handling, use protective clothes, appropriate gloves (plastic, rubber or leather) and protective glasses (EN ISO 16321-1:2022; EN 166).

Wash hands after handling the product and take care of personal hygiene.

8.2.3	Environmental exposure controls

Prevent water contaminated by the product entering the sewer system. The leaked product must be collected.

- SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

9.1.	Information on basic physical and chemical properties

- a) Physical state	liquid

- b) Colour	colourless

- c) Odour	odourless

- d) Melting point/freezing point	crystallization point <-30 C

- e) Boiling point or initial boiling point and boiling range

- ca. 110 - 120 °C

f) Flammability	not combustible (based on molecular structure)

- g) Lower and upper explosion limit		not applicable (non-combustible, non-explosive inorganic material)

h) Flash point		not applicable (not combustible, inorganic)

- i) Auto-ignition temperature	not applicable (not combustible, inorganic)

- j) Decomposition temperature 	ammonium nitrate: >170 °C; urea: >134 °C

- k) pH	7,00,5 (20 °C)

l) Kinematic viscosity	dynamic: 3,96 mPa.s (20 °C)

- m) Solubility	The product mixes with water in all proportions. Solubility in water: ammonium nitrate 1920 g/l; urea: 624 g/1 (20 °C).

- n) Partition coefficient n-octanol/water (log value)	ammonium nitrate: -3.1; urea: -1,73

o) Vapour pressure	6,998 hPa (20 °C) – at 32 % N content

p) Density and/or relative density	1,26 – 1,29 g/cm3

q) Relative vapour density	ca. 1,07 (15 °C)

r) Particle characteristics	not applicable for liquid

9.2.	Other information

- No further data available.

- SECTION 10: STABILITY AND REACTIVITY

10.1.	Reactivity

- The product is stable under normal storage, handling and use conditions.

10.2.	Chemical stability

- The product is stable under normal storage, handling and use conditions.

10.3.	Possibility of hazardous reactions

In case of strong heating it melts and degrades while forming toxic gases, the heating of fertilizer in strong closure (e.g.: in pipes or drains) may lead to violent reactions or explosions, especially if it is contaminated with the materials listed in section 10.5.

In contact with basic substances like lime, ammonia gas is released.

10.4.	Conditions to avoid

Heating to temperature above 170 °C (degradation during gas formation). Vicinity of heat source or fire.

Contamination with incompatible materials (see section 10.3).

10.5.	Hazardous decomposition products

Combustible materials, reducing agents, acids and bases, sulphur, chlorates, chlorides, chromates, nitrites, permanganates, metallic powders and other substances containing metals as copper, nickel, cobalt, zinc, and their alloys.

10.6.	Hazardous decomposition products

Ammonia, nitrogen oxides.

- SECTION 11: TOXICOLOGICAL INFORMATION

11. 1.	Information on hazard classes as defined in Regulation (EC) No 1272/2008

We hereby give information about the results of the conducted toxicological studies (suitable for cross-reference) about the calcium ammonium nitrate fertilizer as well as about pure ammonium nitrate as main component and other nitrates and ammonium salts.

Acute toxicity

| Test substance | CAS number | Exposure route | Species | Result |
| --- | --- | --- | --- | --- |
| Ammonium nitrate | 6484-52-2 | oral | rat | LD50: 2950 mg/kg |
|  |  | dermal | rat | LD50: > 5000 mg/kg |
|  |  | inhalation | rat | LC50: > 88,8 mg/l |
| Urea | 7704-34-9 | oral | rat | LD50: 14 300 mg/kg bw |

Skin irritation

| Test substance | CAS number | Exposure route | Species | Result |
| --- | --- | --- | --- | --- |
| Ammonium nitrate | 6484-52-2 | dermal | rabbit | non-irritant |
| Urea | 7704-34-9 |  |  |  |

Eye irritation

| Test substance | CAS number | Species | Result |
| --- | --- | --- | --- |
| Ammonium nitrate | 6484-52-2 | rabbit | irritant |
| Urea | 7704-34-9 | rabbit | non-irritant |
| Calcium ammonium nitrate (CAN) 77,9% ammonium nitrate content | - | rabbit | non-irritant* |

*: Test results for various NPK fertilizers containing ammonium nitrate and CAN fertilizer containing 77.9% AN indicate that products with less than 80% AN are not eye irritants.

Skin sensitization

| Test substance | CAS number | Species | Result |
| --- | --- | --- | --- |
| Ammonium calcium nitrate dual salt | 15245-12-2 | mouse | not sensitizing |
| Urea | 7704-34-9 | - | not sensitizing |

STOT-repeated exposure

| Test substance | CAS number | Exposure route | Species | Result |
| --- | --- | --- | --- | --- |
| Ammonium sulphate | 7783-20-2 | oral | rat | NOAEL: 256 mg/kg/day (52 week test) |
| Potassium nitrate | 7757-79-1 | oral | rat | NOAEL ≥ 1500 mg/kg/day (28 day test) |
| Ammonium nitrate | 6484-52-2 | inhalation | rat | NOAEC of ≥ 185 mg/m³ |

Carcinogenicity:

Urea is not carcinogen and for ammonium nitrate no data available.

Mutagenicity

| Test substance | CAS number | Test type | Species | Result |
| --- | --- | --- | --- | --- |
| Ammonium calcium nitrate dual salt | 15245-12-2 | Bacterial reverse mutation test | S. typhimurium; E. coli | negative |
|  |  | In vitro mammalian chromosome aberration test | Human peripheric lymphocytes | negative |
| Potassium nitrate | 7757-79-1 | Mammalian gene mutation test | mouse lymphomas | negative |
| Urea | 7704-34-9 | assessed with all three types of tests |  | negative |

Reproductive toxicity

| Test substance | CAS number | Exposure route | Species | Result |
| --- | --- | --- | --- | --- |
| Potassium nitrate | 7757-79-1 | oral | rat | NOAEL:>=1500 mg/kg bw/day |
| Urea | 7704-34-9 | oral | rat | NOAEL: 500 mg/kg bw/day |

Information on likely routes of exposure

The most probable route of exposures is skin and eye exposure, which can be reduced to minimal with the use of PPE. The inhalation exposure is only possible if during the use of the product dust is formed and no sufficient ventilation is available. In case of normal circumstances ingestion is not likely, only accidental ingestion is possible. The possible symptoms are listed in section 4.2.

11.2. Information on other hazards

No further information.

- SECTION 12: ECOLOGICAL INFORMATION

12.1.	Toxicity

When released in large quantities into natural waters, it leads to eutrophication. No toxicological information available for the product. We hereby give information about the results of the conducted toxicological studies (suitable for cross-reference) about pure ammonium nitrate and other nitrates.

| Test substance | CAS number | Test | Species/group of animals | Result |
| --- | --- | --- | --- | --- |
| Ammonium nitrate | 6484-52-2 | Short-term toxicity to fish | carp (Cyprinus carpio) | LC50 (48 h): 447 mg/l |
| Potassium nitrate | 7757-79-1 | Toxicity for invertebrates | daphnia (Daphnia magna) | EC50 (48 h): 490 mg/l |
| Potassium nitrate | 7757-79-1 | Test conducted on algae and aquatic plants | basal diatom algae | EC50 (10 d): > 1700 mg/l |

| Test substance | CAS number | Test | Species/group of animals | Result |
| --- | --- | --- | --- | --- |
| Karbamid | 7704-34-9 | Short-term toxicity to fish | ide (Leuciscus idus) | LC50 (48 h): > 6810 mg/l |
|  |  | Toxicity for invertebrates | water flea (Daphnia magna) | EC50 (24 h): > 10000 mg/l |
|  |  | Test conducted on algae and aquatic plants | Microcystis aeruginos | NOEC: 47 mg/l |

12.2.	Persistence and degradability

Consists of non-persistent, inorganic materials.

Ammonium nitrate completely dissociates into its ions in water. It degrades in the natural nitrification/denitrification cycle. With the help of certain bacteria, ammonium ion transforms to nitrite, then to nitrate even under natural and controlled conditions (sewage treatment plants). Biological degradation time in sewage treatment plants is 52 g N/kg dissolved solid substance/day at 20 °C. Nitrate degrades under anaerobic conditions even under nitrate even under natural and controlled conditions (sewage treatment plants). Products of anaerobic degradation: dinitrogen oxide, nitrogen, ammonia. Biological degradation time in sewage treatment plants is 70 g N/kg dissolved solid substances/day at 20 °C.

Urea breaks down into ammonia and carbon dioxide when exposed to moisture. It is useful as a source of nitrogen (biodegradation: 4mg/l at 20 C in 1 hour).

12.3.	Bioaccumulative potential

Not bioaccumulating, because its components are inorganic materials, and their partition coefficient are low.

12.4.	Mobility in soil

Ions formed during dissolution are mobile; their potential to adsorption is low.

12.5.	Results of PBT and vPvB assessment

Non PBT and vPvB, because its components are inorganic materials.

12.6.	Endocrine disrupting properties

There is no known effect.

12.6.	Other adverse effects

No other adverse effects are known.

SECTION 13: DISPOSAL CONSIDERATIONS

13.1.	Waste treatment methods

Depending on the extent and the type of the contamination, it can be used as fertilizer or can be disposed via licensed waste management company. Recommended codes according to the List of Waste:

06 03 14 solid salts and solutions other than those mentioned in 06 03 11 and 06 03 13

15 02 03 absorbents, filter materials, wiping cloths and protective clothing other than those mentioned in 15 02 02

Information regarding the disposal of the packaging

The sacks, containers which are thoroughly cleaned with water - with the permission of the local authorities - can be disposed or recycled as non-hazardous waste (Do not remove the label from the container before cleaning). Recommended code according to the List of Waste:

15 01 02 plastic packaging

SECTION 14: TRANSPORT INFORMATION

14.1. 	UN number or ID number: Not dangerous goods.

14.2.	UN proper shipping name: Not dangerous goods.

14.3. 	Transport hazard class(es): Not dangerous goods.

14.4.	Packing group: Not dangerous goods.

14.5.	Environmental hazards: not hazardous for the environment

14.6.	Special precautions for user: not required

14.7. 	Maritime transport in bulk according to IMO instruments:

not applicable

SECTION 15: REGULATORY INFORMATION

15.1.	Safety, health and environmental regulations/legislation specific for the substance or mixture

| Directive (EU) No 2012/18 (SEVESO III.) on the control of major-accident hazards involving dangerous substances | - |
| --- | --- |
| Regulation (EU) 2019/1009 laying down rules on the making available on the market of EU fertilising products | Liquid, inorganic fertilisers containing a single macroelement |
| Regulation 1907/2006/EC concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), restrictions according to Annex XVII | The product is not subject to restriction. |
| Regulation 1907/2006/EC concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), authorization | The product does not contain any substances of very high concern. |
| Regulation (EU) 2019/1148 on the marketing and use of explosives precursors | The product is not subject to the regulation. |

15.2.	Chemical safety assessment

Chemical safety assessment has been performed for ammonium nitrate and urea.

SECTION 16: OTHER INFORMATION

Important changes in the safety data sheet:

The safety data sheet has been adapted to Regulation (EU) 2020/878.

Abbreviations:

LD50 	– Lethal dose resulting in 50 % mortality

EC50 	– Effective concentration, 50 %

DNEL	– Derived-No-Effect-Level

LC50 	– Lethal concentration resulting in 50 % mortality

NOAEL	– No Observed Adverse Effect Level

NOAEC	– No Observed Adverse Effect Concentration

PBT 	– Persistent, bioaccumulative and toxic

vPvB 	– very persistent and very bioaccumulative

Method of evaluation of data:

Test results for substances or materials for cross-referencing, as well as the method based on the general concentration limits for mixtures set out in Annex I of CLP.

The product is not subject to ADR/RID (special provision 307), it is non-oxidizing.

Based on eye irritation studies conducted by the Harlan laboratories Ltd on different fertilizers containing ammonium nitrate (CAN27, NPT fertilizers), fertilizer mixtures containing less than 80 % ammonium nitrate are not considered to be eye irritant.

Important references:

- Chemical safety report for ammonium nitrate, 2016

- International chemical safety cards ICSC 0216, 2001

- Hommel: Hazardous substances, 1989

- Harlan Laboratory: Report for CAN 27 in vivo testing, Report no. D36408, 2011

- Fertilizers Europe: Assessment of ammonium nitrate based fertilizers as eye irritant for classifiation purposes, 2011